# BRICKWEBB

PART 1 – GENERAL

  - SECTION INCLUDES:

- Assembled sheets of BRICKWEBBTM to be used in the aiding of thin brick installation.

  - DELIVERY, STORAGE AND HANDLING:

- Deliver brick, sheets, accessories and loose trim in manufacturer’s original packaging.

- Store all BRICKWEBB, Sheets, accessories, and trim off the ground covered with waterproof covering to protect from weather and store at temperatures between 95◦ and 55◦ Fahrenheit.

# PART 2 - PRODUCTS

  - SYSTEM:

- Manufacturer:  Old Mill Brick, Inc. (801) 542-7050

  - MATERIALS:

A.  BRICKWEBBTM

B.	Brick

    - ASTM C1088-04, Type TBS, Grade Exterior

    - Special shapes, corners, trim pieces and loose brick slabs.

C.	Adhesive

Recommended Polymer Modified Thin-set Mortars and Thin-set Pre-mixed adhesives.

D.	Grouting Mortar

Type S Mortar Mix or Sanded Tile Grout.

  - MISCELLANEOUS ACCESSORIES:

- Cleaning compound ProSoCo SureKlean

  - No. 101 Lime Solvent for red and dark brick.

  - No. 600 Detergent for tan and pink brick.

  - Vana-Trol for White, gray or brown brick and brick subject to metallic staining.

# PART 3 - EXECUTION

  - EXAMINATION:

- Verify that substrate is sound, smooth and without projections that will interfere with installation of BRICKWEBBTM.

- Beginning of installation means acceptance of existing conditions.

  - INSTALLATION:

Before you begin check all official building codes. Once the substrate is accepted you are ready to start laying out your project. Start by chalking horizontal lines every 21 ½” this is equal to two rows of BRICKWEBB. If there are corner walls within your project you will begin here first. Start by applying adhesive using a 3/8” by 3/8” notch trawl. Next, apply corners at the top of your wall by pressing firmly against BRICKWEBB corner sheets ensuring adhesive penetrates thru webbing. Only go down two sheets, approximately 42”. Next, apply adhesive to top section of your flat wall. Apply BRICKWEBB sheets by overlapping exposed webbing. Firmly press in place ensuring adhesive extrudes through webbing. Once your wall is covered, let set for 24 hours. Next, apply grout by using ether a grout bag if using type S mortar or tile flout if using sanded tile grout.  Apply in moderately sized areas as to prevent drying. Next, tool area, once grout becomes semi firm, to desired look and texture.

- Cleaning

      - Brush grout joint with stiff bristle brush using diagonal strokes, when grout becomes stiff, to remove excess grout from brick faces and joints.

      - Clean brick as required, after grout is cured, in accordance with cleaning compound manufacturer’s instruction.

      - Rinse brick promptly and thoroughly with clean water.

      - Use appropriate cleaning solution 24 to 72 hours after grouting.

- Sealants

  - Sealant should be a high performance penetrating sealer; check with your local building supply professional.

END OF SECTION